### Observing Moisture		     Name______________________

### Using a Psychrometer

A psychrometer measures the amount of  relative humidity in the air. Relative humidity is the amount of moisture in the air divided by the maximum            amount the air can hold. The higher the relative humidity is,                the more likely it is to rain.

Can you use your psychrometer to predict a storm?

### Materials:  2 thermometers, small piece of wet gauze, rubber banc, fan, relative humidity table

### Instructions

- TAPE two thermometers to the surface of a table with the numbers facing up and the liquid filled ends sticking over the edge of the table about 2.5 cm (1 inch).

- Using the rubber band, TIE a small piece of  wet gauze around the liquid filled end of one thermometer.

- BLOW a fan on the thermometers until the temperature stops falling.

- WRITE DOWN the temperature on both thermometers.

- SUBTRACT the temperature on the wet thermometer from that of the dry one.

- LOOK at the table below. FIND the dry thermometer temperature on the left and FOLLOW it to the right. FIND the difference between the two temperatures on the top, and FOLLOW it down. The number where the row and column intersect is the relative humidity.

### Relative Humidity Table (in percent)

Dry Bulb        Dry Bulb Minus Wet Bulb (degrees Celsius)

| °C | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 88 | 77 | 66 | 55 | 44 | 34 | 24 | 15 | 6 |  |
| 11 | 89 | 78 | 67 | 56 | 46 | 36 | 27 | 18 | 9 |  |
| 12 | 89 | 78 | 68 | 58 | 48 | 39 | 29 | 21 | 12 |  |
| 13 | 89 | 79 | 69 | 59 | 50 | 41 | 32 | 22 | 15 | 7 |
| 14 | 90 | 79 | 70 | 60 | 51 | 42 | 34 | 25 | 18 | 10 |
| 15 | 90 | 81 | 71 | 61 | 53 | 44 | 36 | 27 | 20 | 13 |
| 16 | 90 | 81 | 71 | 63 | 54 | 46 | 38 | 30 | 23 | 15 |
| 17 | 90 | 81 | 72 | 64 | 55 | 47 | 40 | 32 | 25 | 18 |
| 18 | 91 | 82 | 73 | 65 | 57 | 49 | 41 | 34 | 27 | 20 |
| 19 | 91 | 82 | 74 | 65 | 58 | 50 | 43 | 36 | 29 | 22 |
| 20 | 91 | 83 | 74 | 67 | 59 | 53 | 46 | 39 | 32 | 26 |
| 21 | 91 | 83 | 75 | 67 | 60 | 53 | 46 | 39 | 32 | 26 |
| 22 | 91 | 83 | 76 | 68 | 61 | 54 | 47 | 40 | 34 | 28 |
| 23 | 92 | 84 | 76 | 69 | 62 | 55 | 48 | 42 | 36 | 30 |
| 24 | 92 | 84 | 77 | 69 | 62 | 56 | 49 | 43 | 37 | 31 |
| 25 | 92 | 84 | 77 | 70 | 63 | 57 | 50 | 44 | 39 | 33 |